# Internet Guidance

Exported from Holy-Writings.com on 2026-06-19 - 1 clipping

Source: Bahá'í Library Online (bahai-library.com), curated by Jonah Winters. Used by permission of the curator. Original citation: Bahá'í Internet Agency, Internet Guidance, bahai-library.com.

──────────────────────────────────────────────────────────────────────

Podcasting: A Technical Primer

Bahá’í Internet Agency

2006

Summary

This guide is intended as a supplement to the document “Blogging and the Bahá’í Faith:

Suggestions and Possible Approaches,” for those interested in podcasting, which is

essentially an audio version of the text-based blog.

The Internet is a wonderful combination of wide reach and low costs. Even the most

powerful radio transmitter cannot reach a fraction of the potential listeners possible via

the Internet. Yet, even the smallest radio station is far more expensive than a typical

podcasting operation. In many parts of the world, radio continues to be the most common

form of mass communications and the audio content from podcasts can easily be shared

with the radio world.

Content is King

Listeners will forgive many technical errors if the content is interesting to them but the

best produced content will not be effective if the content is weak. It is easy to get so

immersed in the production process that one loses track of the listener’s interests. Don’t

let technical issues stop you from recording creative or valuable information. Podcasts

provide a unique vehicle to share first person testimonies of faith, reflections about the

Bahá’í teachings, or educational material. Unlike text, they allow the powers of the

human spirit to be more directly experienced by listeners.

Preparing for Recording

Audio recording can easily go wrong. A microphone placed next to a fan or an open

window can pick up extraneous noise. Accidentally setting the recording levels too low

can result in the recording being barely audible. These problems and most others can be

avoided simply by running a test.

When recording a live event, it is best to have backups of key items. Even a well-tested

setup can fail for a variety of reasons. The speaker may break the microphone or the

batteries may die. It’s always a good idea to be over prepared.

Production Masters

There are two basic rules of thumb when creating content: you often will have more

content than you need, and the final content will be of a lesser quality than that of the

original recording. It is always a good idea to record at the highest possible recording

standards. The original recording is called the master. The master is used to make the

audio files that you will actually distribute. While you may intend to use the content for a

low-quality Internet audio file, having a high-quality master will allow you to produce a

CD-ROM and higher-quality audio files at a later date.

There are two types of recording formats: lossless and lossy. Lossless means the format

records all the audio content it hears. Lossy, on the other hand, is a format where file

sizes are greatly reduced by intelligently removing “unnecessary” audio information. A

master normally uses a lossless format, such as wav and flac, and then the final product is

exported to a lossy format, such as mp3 and vorbis.

If you are going to be extensively editing your content, keep a book to track your

recordings. That means labeling the tape, CD-ROMs, or file names of the recording

medium. Most recording systems provide some sort of counter or timer that allow you to

identify the exact starting and stopping points for a segment. Not only will you want to

record the segment times, but you may want to note points of interest during the

recording just in case you want to extract a specific comment.

Hardware and Software

Recording audio has never been easier. There are many good ways to record audio.

Below is a list of some workable systems 1 .

•   MiniDisk Recorder (such as Sony’s MZ-R10). These are often used by

journalists because they are small but make high-quality recordings. Because the

recording is in digital format, it can be copied directly to a computer.

•   Computer. Computers – laptops and desktops – have the ability to record.

Computers have an advantage in being able to monitor the recording via computer

screens. Computers are best when recording from an audio device, such as a tape

or CD-ROM. This may not work as well when recording people talking because

the computer may get in the way of the interview.

•   Cassette tape. Some of the higher quality units can do a good job of recording.

The main problem is that the recording will still have to be digitized. This doubles

the amount of time required because the digitization process takes as long as the

recording is long. So a sixty minute recording requires another sixty minutes to

convert.

•   MP3 Device. Some MP3 music players can record at high fidelity. Many MP3

devices have built in recording capabilities. Check the device for its recording

specifications. Some recorders are limited recording speech because they don’t

want to be used for recording music.

•   Phone. It’s possible to use a phone to record audio. You could call your home

answering machine or use an online service. The recording probably will not be

that good, but it’s better than losing an opportunity.

http://www.pwop.com/podcastingkit.aspx Here is a sample of a fairly high-end podcasting kit. This is for

serious work and is not required to do good work.

•   Digital camera. If you have a digital video camera, you can use the camera’s

built-in microphone jack to record audio. Don’t use the built-in microphone. The

microphone’s placement in the camera can introduce noises associated with the

camera’s internal mechanisms.

•   VCR Deck. You may find yourself in a situation where you need a high-quality

recording device but you did not bring one, or the one you brought had some

problem at the last minute. The old VCR actually records with great fidelity.

Finding a unit and a blank tape is often easy. Like the cassette tape, you are going

to have to digitize the recording in order to get a computer file.

•   Microphone. The microphone is usually the most important element for getting

good sound. A suitable microphone will probably cost between $50 and $150. For

purely voice, an $80 microphone should be fine 2 . You may need more than one

microphone if you are recording individuals and groups. Some microphones are

good for a single speaker while other microphones are designed to pick up all the

sound in a room.

•   Accessories. A few small items can make a big difference. Below are a few items

to consider:

o A pop filter blocks the sounds of wind and strongly aspirated letters, such

as the letter P.

o Microphone stands can keep a microphone from falling or being bumped.

o Gaffing tape can secure a cable on the floor so that no one trips over it –

without damaging the surface with sticky residue.

In general, if you are traveling to a location to record an event, a MiniDisc or MP3 device

is probably the best solution. If you are working with currently existing content, the

computer is the way to go.

Audio software is an important ingredient in the recording process. If you are using a

computer, recording software is required to turn the audio content into a usable computer

file. Once you have a computer file, you still have to cut, splice, and manipulate the

recordings. Cutting is almost always required. The time between the start of the recording

and the actual start of the audio content needs to be trimmed. Less obvious is the need to

cut out a small interlude between speakers or the sound of the recording unit being turned

on and off. This last issue is especially common if you are working with an audio file

recorded from a cassette tape.

Splicing is where two audio segments are put together. This may be due to your using

two audio disks for a long talk, or you may simply find some content not as interesting

and thus you cut it out. Splicing is also frequently done when the interviewer wants to

insert their questions. This is done because the interviewer may not have their own

microphone during the recording. The interviewer’s questions are recorded at a later

point and then inserted before each answer during the editing process. This is how most

professional interviews are performed.

http://www.geekfishing.net/internet/podcast/2005/03/30/podcast_microphone.html Here is a description

of various microphones suitable for podcasting.

There are many ways to manipulate audio content. The most common reason to

manipulate the content is to remove unwanted noise, such as pops, hisses, and hums. This

is often a problem with recordings originating from old cassette tapes. Another common

technique is to adjust or normalize the audio levels so that the recordings sound

essentially the same in terms of loudness within the recording and between different

recordings. You don’t want to force the listener to constantly adjust the volume level up

and down as the sound level changes. Sometimes, adjusting the tone or adding reverb can

dramatically affect the sound quality of the tape. Naturally, the better your original

recording, the less adjustments you have to do later.

Producing for the Internet

The fundamental conundrum with preparing content for the Internet is that the better the

quality of the content, the bigger the file size and thus the fewer people who will be able

to access it. As file sizes go down, so does the quality level. People may be capable of

getting the content, but choose not to because of its poor quality. Below are a few factors

to consider.

1. Type of recording. A music file clearly requires better production values than

would a voice recording.

2. Audience bandwidth. If your audience has broadband connections, you can offer

larger files than you might do with an audience using lower-bandwidth

connections.

3. Distribution model. Streaming, file downloads, and RSS are all common ways to

get audio files to the listener. Each one brings with it a set of bandwidth

considerations. Podcasting uses RSS, and in most cases this is the optimal transfer

mechanism.

4. Player type. If you are fairly sure that most of the listeners will use their

computer’s ten-dollar speakers, you don’t have to be as concerned with sound

quality as you would if the recording is going to be played on a stereo or MP3

player.

Recording Tags

When you convert your file to the MP3 format – a common end-user audio format – there

will be the option to provide a myriad of information about the recording and how it was

produced. Enter meaningful information about the genre and style of the recording. This

information will appear in the listener’s audio directory. Provide titles that are concise

and meaningful. A title that starts with “The Baha’i General Information Podcast: Raising

Healthy Children” will probably result with the user’s display showing the first three

words. So the user sees ten recordings that all say “The Baha’i General…”. Put your

production’s name in the author field and put “Raising Healthy Children” in the title

field. Put in copyright information. Even if the information you are producing does not

have a copyright, your production of that content can be copyrighted. You are of course

free to let others know that the content may be freely used as part of your copyright

statement. It also does not hurt to put in the name of the locations where the recordings

can be found. You never know when someone will receive the recording from a third-

party and want to discover the source of the file. In fact, you cannot assume that your

content will not be retrieved from some other source. Therefore, you may want to have a

short introduction at the start or end of the recording identifying yourself, the topic title,

and the source of the recording.

Tags and/or descriptions come into play when you list your recordings on search engines

and in podcast directories, such as iTunes. Accuracy is very important for getting the

right people to your content.

Finding a Home

While audio recordings are small in comparison with video files, they are large when

compared with text. A standard web site may not be able to host a large number of audio

files. You may run out of disk space or the service provider may charge you more for the

extra bandwidth your site is using.

There are a number of free sites that will host your recordings. Below are some well-

known free hosting sites.

•   PodServe (http://podserve.biggu.com/)

•   Odeo (http://odeo.com/)

•   OurMedia (http://www.ourmedia.org/)

OurMedia is a free service, but it has performance issues and its requirement for public

domain content may prevent some content providers from uploading their materials. It is

certainly a good service when there is no funding for anything else. At some point

OurMedia will probably become more dependable.

Combining Old and New

Remember that podcasting is a combination of old technologies, such as radio, and new

technologies, such as the Internet and RSS. The recording and production of the masters

is in principle very similar to what people have been doing in radio for decades. The

lessons from radio and music production are well worth learning. The newer aspects are

still in flux. The guidelines here are far less clear and the only recommendation is to

experiment and be ready for new systems and processes.

— Internet Guidance (Used by permission of the curator)